APVMA risk assessment manual

Environment

APRIL 2019

© Australian Pesticides and Veterinary Medicines Authority 2019

ISSN	2652-0583 (electronic)

Ownership of intellectual property rights in this publication

Unless otherwise noted, copyright (and any other intellectual property rights, if any) in this publication is owned by the Australian Pesticides and Veterinary Medicines Authority (APVMA).

Creative Commons licence

With the exception of the Coat of Arms and other elements specifically identified, this publication is licensed under a Creative Commons Attribution 4.0 Australia Licence. This is a standard form agreement that allows you to copy, distribute, transmit and adapt this publication provided that you attribute the work.

A summary of the licence terms is available from www.creativecommons.org/licenses/by/3.0/au/deed.en. The full licence terms are available from www.creativecommons.org/licenses/by/3.0/au/legalcode.

The APVMA’s preference is that you attribute this publication (and any approved material sourced from it) using the following wording:

Source: Licensed from the Australian Pesticides and Veterinary Medicines Authority (APVMA) under a Creative Commons Attribution 4.0 Australia Licence.

In referencing this document the Australian Pesticides and Veterinary Medicines Authority should be cited as the author, publisher and copyright owner.

Use of the Coat of Arms

The terms under which the Coat of Arms can be used are set out on the Department of the Prime Minister and Cabinet website (see www.dpmc.gov.au/pmc/publication/commonwealth-coat-arms-information-and-guidelines).

Disclaimer

The material in or linking from this report may contain the views or recommendations of third parties. Third party material does not necessarily reflect the views of the APVMA, or indicate a commitment to a particular course of action. There may be links in this document that will transfer you to external websites. The APVMA does not have responsibility for these websites, nor does linking to or from this document constitute any form of endorsement. The APVMA is not responsible for any errors, omissions or matters of interpretation in any third-party information contained within this document.

Comments and enquiries regarding copyright:

Assistant Director, Communications

Australian Pesticides and Veterinary Medicines Authority

PO Box 6182

KINGSTON ACT 2604 Australia

Telephone: +61 2 6210 4988

Email: communications@apvma.gov.au.

This publication is available from the APVMA website.

Contents

1	environmental risk assessment	4

1.1	Step 1—Problem formulation	4

1.2	Step 2a—Environmental exposure assessment	4

1.3	Step 2b—Environmental hazard assessment	5

1.4	Step 3—Environmental risk characterisation	5

1.5	Refining the risk assessment	6

2	Regulatory Framework	7

3	Key fate endpoints	8

4	Combined residues	10

5	Overall conclusions	16

6	Labelling recommendations	17

7	References	18

Abbreviations	19

List of tables

Table 1:	Key regulatory endpoints for exposure assessment	9

Table 2:	Toxicity of combined residues to non-target species	11

# environmental risk assessment

The assessment of risk to the environment, like the assessment of risk to human health, is a three-step process. A Hazard Assessment determines how toxic a chemical is to non-target animals and plants an Exposure Assessment involves understanding what happens to a chemical once it enters the environment, and Risk Characterisation uses information from the first two steps combined to determine the overall level of risk.

This document intends to provide guidance on how to conduct a risk assessment for non-target species in the context of the review of pesticide and veterinary applications to the APVMA. The document is a working document, which will be updated to take on board scientific advances and expanded guidance as the necessity arises.

## Step 1—Problem formulation

The objective of the problem formulation phase is to define the scope of the environmental assessment. The use pattern is critical in determining what potential groups of non-target species could be exposed that require assessment. For existing active constituents, the APVMA determines which previous decisions can be relied on in relation to a reference product, and what new risks require assessment. The formulation type, the method of application/administration, the mode of action of the active constituent, and its behaviour in the treated crop/animal are important considerations in determining the scope of the environmental assessment.

## Step 2a—Environmental exposure assessment

Once a chemical enters the environment a lot of things can happen to it. It can be moved around by air or water or come to rest in soil. It can be broken down by sunlight, water or microorganisms. It can also be taken up by plants and animals, where it can either be metabolised (broken down by an organism) or bioaccumulate (stored in an organism’s tissues). Scientists take all these factors into account when conducting studies to determine the behaviour, or fate, of a chemical in the environment.

A chemical’s fate also depends upon how it is used. For example, a chemical delivered through a spray mechanism may behave differently from one added directly to soil or to one fed to an animal. When conducting environmental exposure assessment scientists consider factors such as the method of application, the target crops or animals, what time of the year it is usually applied and the geographic area in which it will be used.

The final product of an environmental exposure assessment is the calculation of a Predicted Environmental Concentration (PEC). A PEC is an estimated value of how much of a chemical (and its break-down components) is likely to be found in a particular part of the environment, such as water or soil or sediment, as a result of normal use. PECs are estimated using standard models that consider the application rate(s), chemical and environmental fate properties, including the dissipation/metabolism of the active constituent between applications/treatments. This value provides one part of the information needed to establish the overall level of risk, or Risk Characterisation.

## Step 2b—Environmental hazard assessment

Some chemicals are more toxic than others. The purpose of the environmental hazard assessment is to determine the hazard to non-target plants and animals of a chemical, based on its toxicity.

Toxicity is generally established in controlled laboratory settings. Laboratory studies determine the effects of short and long-term exposure to particular chemicals on small numbers of selected animal and plant species, including birds, insects, earthworms and water-based plants and animals such as fish and aquatic invertebrates. Such tests typically use worst-case conditions, eg sensitive life stages and constant exposure. Appropriate end-points from mammalian toxicology data are used to support the wild mammal assessment.

Regulatory bodies around the world assess toxicity tests using the same general approach. Because of this harmonized approach for testing, registrants are often able to conduct one mutually acceptable study that satisfies global requirements.

When determining the toxicity of a particular chemical, scientists are interested in determining the lethal level of short-term exposure and also the longer-term impacts on growth, development and reproduction. Some more complex studies investigate the impact on the composition of a species, on an ecological community and on an entire ecosystem.

Ecotoxicity endpoints used in the risk assessments are adjusted to determine regulatory acceptable concentrations (RACs) to account for potential differences in species sensitivity as well as varying protection goals (ie protection at the community, population, or individual level). This value provides the other part of the information needed to establish the Risk Characterisation.

## Step 3—Environmental risk characterisation

Risk characterisation is the final step in the risk assessment process. In this step, the results of the first two steps are combined to produce an estimate of the overall risk to the environment of a particular chemical.

There are a number of different methods available to determine the overall level of risk. The assessment usually begins at a ‘screening level’ that assumes the worst-case scenario of direct exposure to the maximum possible exposure concentration, dose or rate, in order to identify those substances and associated uses that do not pose a risk. The screening level assessment employs a deterministic approach, so-called because it determines a single numeric value known as a Risk Quotient, or RQ, which compares the PEC (derived in Step 1) to the regulatory acceptable value (derived in Step 2).

Some chemicals have the potential to be highly toxic and persistent in the environment for a very long time. Particular care is required when assessing persistent, bioaccumulative and toxic (PBT) chemicals, whose effects on the environment are often apparent only after a prolonged period of time. Australia has international obligations when assessing chemicals, including the Stockholm Convention on Persistent Organic Pollutants (POPs). The APVMA takes these obligations very seriously and assesses each Agvet chemical with respect to its persistence in the environment, the ability of the chemical to bioaccumulate and its toxicity to environmental organisms.

## Refining the risk assessment

The APVMA can require additional studies to be conducted on a product. Additional studies are aimed at developing a more accurate understanding of the risks to the environment from a particular chemical product, which in turn allows regulators to establish more effective rules for its use. For example, a field study may be requested to determine whether or not a chemical behaves the same way ‘in nature’ as it does in a laboratory. Or an ecotoxicology test may be requested to ascertain if the impact of a chemical on one or two species in the laboratory is applicable to the impacts that might be seen on a whole ecosystem.

# Regulatory Framework

The Agricultural and Veterinary Chemicals Code (Agvet Code), provides the basis for using risk analysis to regulate activities with agricultural and veterinary chemicals (Agvet chemicals) in Australia.

The objective of the Code is for the evaluation, approval, and control of the supply, of active constituents for proposed or existing agricultural chemical products or veterinary chemical products; and the evaluation, registration, and control of the manufacture and supply, of agricultural chemical products and veterinary chemical products.

The Act mandates that the APVMA implement the Code in a manner, amongst other things, that

recognises that the health and safety of human beings, animals and the environment is the first priority of the regulatory system

reflects established best-practice principles for the assessment and management of risk, based on science

balances regulatory effort and any burden with the risk of the use of the products and constituents to the health and safety of human beings, animals and the environment.

In considering the environmental safety of the proposed use of a product, the APVMA must have regard to the toxicity of the active constituent and its residues, including metabolites and degradation products, in relation to relevant organisms and ecosystems. The APVMA must also be satisfied under s14 of the Agricultural and Veterinary Chemicals Code Act 1994 that the proposed use of the product meets the environmental safety criteria with respect to s5A(1)(c), and the labelling criteria under s5D(1) (or s112(2)(d) for permits).

# Key fate endpoints

The assessment should identify the key regulatory endpoints for use in the environmental exposure assessment.

For screening level assessments for products multiple times in one season, the predicted environmental concentration (PEC) is calculated assuming non-target species are exposed to the peak concentration immediately after the last application. Dissipation of the active constituent between applications is considered. The following equation is used to calculate the cumulative rate used in the exposure assessment.

Cumulative rate = Single rate (1-EXP(-N*ln2/DT50*interval))/(1-EXP(-ln2/DT50*interval))

in which:

Cumulative rate = accumulated application rate immediately after the last application (g ac/ha)

Single rate = single application rate (g ac/ha)

N = number of applications

DT50 = half-life in relevant environmental compartment

Interval = time between application (d)

For assessment of runoff risks to aquatic species, and risks to soil organisms and pre-emergent exposure to non-target terrestrial plants, a soil DT50 value is used in the calculation of the cumulative exposure rate. Typically the longest field DT50 is used for screening level assessments; however, the most appropriate value is determined using expert judgement based on the available data and the tier of assessment.

For the assessment of terrestrial vertebrates, bees & other non-target arthropods, the typically a default DT50 value of 10 days is applied for dissipation on foliage and/or food items. Dissipation data can be used to refine the assessment if it is shown that the dissipation of the active constituent is faster than assumed.

For assessment of aquatic species (screening level and spray drift assessments), a water phase or whole water/sediment system DT50 value is typically used. For assessment of sediment dwelling species (screening level and spray drift assessment), whole water/sediment system DT50 is typically used.

When a runoff assessment is required, it is also necessary to consider adsorption parameters to soil and sediment. For a screening level assessment, the predicted Kd value for the soil of 1 per cent organic carbon is used (and 5 per cent organic carbon when predicting adsorption to sediment) using a regression analysis of the available data. Higher tier assessments would consider more realistic Kd values based on the region or that appropriate for the crop/site being assessed.

Table 1:	Key regulatory endpoints for exposure assessment

| Compartment | Value | Source |
| --- | --- | --- |
| Foliage/food items | DT50Xd | eg default |
| Soil | DT50Xd | eg longest field half-life from eight sites |
|  | KdX mL/g | eg predicted for 1% OC based on regression |
| Water | DT50Xd | eg longest water phase value from two water/sediment systems |
| Sediment | DT50Xd | eg geomean whole system value from two water/sediment systems |
|  | Kp X mL/g | eg predicted for 5% OC based on regression |
| Air | eg Not relevant. Not volatile. |  |

# Combined residues

For new combinations of active constituents, whether as a mandatory tank mix or in formulation, short-term risks of direct exposure to combined residues to non-target species immediately after application are assessed.

Wild mammals and birds could be exposed to combined residues if over-sprayed food sources are consumed immediately after application. Aquatic species and non-target terrestrial plants could be exposed after application as a result of spray drift. Bees could be exposed to combined residues when visiting over-sprayed plants in bloom during treatment, or immediately after application. Similarly, other beneficial (predatory and parasitic) arthropods could be exposed to combined residues on treated plants during or immediately after treatment. Soil organisms (macro- and micro-organisms) could be directly exposed to combined residues in over-sprayed soil within the treatment area.

Endpoints are obtained from formulation toxicity data provided. In the absence of formulation toxicity data, combination toxicity is estimated assuming additive toxicity for organisms. The method for predicting the toxicity value for combined residues follows a pragmatic approach using the concentration addition model detailed by Altenburger et al. (2013). It is assumed that the toxicity of the mixture is attributed to the active constituents. Where one active constituent is calculated to contribute to >90 per cent of the toxicity of combined residues, then risks of combined residues are considered to be no greater than the individual active constituents.

Predicted formulation toxicity values can also be calculated to compare with the toxicity studies provided as to provide validation of the predicted values. EFSA (2013) provides rationale for checking the plausibility of the measured formulation toxicity against the calculated mixture toxicity. This is defined as the model deviation ratio (MDR) where the ratio of the calculated value is divided by that of the measured value. In interpreting the MDR, if the value falls between 0.2 and 5, CA is assumed to hold for the mixture. Where the MDR is >5, the toxicity of the mixture is considered more than additive, and where the MDR is <0.2, the toxicity of the mixture is considered to be less than additive.

Table 2:	Toxicity of combined residues to non-target species

| Fraction in combination | Active 1 | Active 2 | Combined residues |
| --- | --- | --- | --- |
|  | 0.XX | 0.XX | 1.00 |
| Acute toxicity to mammals | LD50 XX mg ac/kg bw Test item Test species Reference | LD50 XX mg ac/kg bw Test item Test species Reference | Measured: LD50 XX mg acs/kg bw Test item Test species Reference Predicted: LD50 XX mg acs/kg bw Relative toxicity contributions: X% + X% MDR X |
| Acute toxicity to birds | LD50 XX mg ac/kg bw Test item Test species Reference | LD50 XX mg ac/kg bw Test item Test species Reference | Measured: LD50 XX mg acs/kg bw Test item Test species Reference Predicted: LD50 XX mg acs/kg bw Relative toxicity contributions: X% + X% MDR X |
| Acute toxicity to fish | LC50 XX mg ac/L Test item Test species Reference | LC50 XX mg ac/L Test item Test species Reference | Measured: LC50 XX mg acs/L Test item Test species Reference Predicted: LC50 XX mg acs/L Relative toxicity contributions: X% + X% MDR X |
| Acute toxicity to aquatic invertebrates | EC50 XX mg ac/L Test item Test species Reference | EC50 XX mg ac/L Test item Test species Reference | Measured: EC50 XX mg acs/L Test item Test species Reference Predicted: EC50 XX mg acs/L Relative toxicity contributions: X% + X% MDR X |
| Toxicity to algae | ErC50 XX mg ac/L Test item Test species Reference | ErC50 XX mg ac/L Test item Test species Reference | Measured: ErC50 XX mg acs/L Test item Test species Reference Predicted: ErC50 XX mg acs/L Relative toxicity contributions: X% + X% MDR X |
| Toxicity to aquatic plants | EC50 XX mg ac/L Test item Test species Reference | EC50 XX mg ac/L Test item Test species Reference | Measured: EC50 XX mg acs/L Test item Test species Reference Predicted: EC50 XX mg acs/L Relative toxicity contributions: X% + X% MDR X |
| Oral toxicity to bees | LD50 XX µg ac/bee Test item Test species Reference | LD50 XX µg ac/bee Test item Test species Reference | Measured: LD50 XX µg acs/bee Test item Test species Reference Predicted: LD50 XX µg acs/bee Relative toxicity contributions: X% + X% MDR X |
| Contact toxicity to bees | LD50 XX µg ac/bee Test item Test species Reference | LD50 XX µg ac/bee Test item Test species Reference | Measured: LD50 XX µg acs/bee Test item Test species Reference Predicted: LD50 XX µg acs/bee Relative toxicity contributions: X% + X% MDR X |
| Toxicity to predatory arthropods | LR50 XX g ac/ha Test item Test species Reference | LR50 XX g ac/ha Test item Test species Reference | Measured: LR50 XX g acs/ha Test item Test species Reference Predicted: LR50 XX g acs/ha Relative toxicity contributions: X% + X% MDR X |
| Toxicity to parasitic arthropods | LR50 XX g ac/ha Test item Test species Reference | LR50 XX g ac/ha Test item Test species Reference | Measured: LR50 XX g acs/ha Test item Test duration + medium Test species Reference Predicted: LR50 XX g acs/ha Relative toxicity contributions: X% + X% MDR X |
| Acute toxicity to soil macro-organisms | LC50 XX mg ac/kg dry soil Test item Test species Reference | LC50 XX mg ac/kg dry soil Test item Test species Reference | Measured: LC50 XX mg acs/kg dry soil Test item Test duration + medium Test species Reference Predicted: LC50 XX mg acs/kg dry soil Relative toxicity contributions: X% + X% MDR X |
| Toxicity to soil micro-organisms | NOEC XX mg ac/kg dry soil (indicate % effect at LOEC or <25% effect at limit dose) Test item Soil process Reference | NOEC XX mg ac/kg dry soil (indicate % effect at LOEC or <25% effect at limit dose) Test item Soil process Reference | Measured: NOEC XX mg acs/kg dry soil (indicate % effect at LOEC or <25% effect at limit dose) Test item Soil process Reference Predicted: NOEC XX mg acs/kg dry soil Relative toxicity contributions: X% + X% MDRX |
| Effects of seedling emergence | ER50 XXg ac/ha Test item Tier of testing Test species (or #species tested if >value) Reference | ER50 XXg ac/ha Test item Tier of testing Test species (or #species tested if >value) Reference | Measured: ER50 XXg ac/ha Test item Tier of testing Test species (or #species tested if >value) Reference Predicted: ER50 XX mg acs/kg bw Relative toxicity contributions: X% + X% MDR X |
| Effects on vegetative vigour | ER50 XX g ac/ha Test item Tier of testing Test species (or #species tested if >value) Reference | ER50 XX g ac/ha Test item Tier of testing Test species (or #species tested if >value) Reference | Measured: ER50 XXg ac/ha Test item Tier of testing Test species (or #species tested if >value) Reference Predicted: ER50 XX mg acs/kg bw Relative toxicity contributions: X% + X% MDR X |

# Overall conclusions

Overall conclusion statements for each of the potential environmental impacts are presented, with appropriate restraints and protection measures that enable a conclusion of acceptable risk (or otherwise) to non-target species. This includes:

fate and transport considerations

effects and associated risks to:

terrestrial vertebrates (including birds and mammals)

aquatic species (including fish, invertebrates, algae and aquatic plants)

bees

other beneficial (predatory and parasitic) arthropods

soil organisms (macro- and micro-organisms)

non-target terrestrial plants.

# Labelling recommendations

Recommendations for labelling will be based on the outcomes of the risk assessments, reference product labels and current labelling standards. The environmental assessment does not consider storage conditions of the product; however, appropriate disposal statements will be recommended as per the Ag and Vet labelling codes.

# References

Altenburger R, Arrehenius A, Backhaus T, Coors A, Faust M, Zitzkat D, 2013, Ecotoxicological combined effects from chemical mixtures, Helmhotz-Zentrum für Umweltforschung GmbH – UFZ, Dept. Bioanalytische Ökotoxikologie, Permoserstr. 15, Lepizig.

EFSA, 2013. Guidance on tiered risk assessment for plant protection products for aquatic organisms in edge-of-field surface waters. EFSA Journal 2013;11(7):3290.

SCEW, 2009. Environmental Risk Assessment Guidance Manual for agricultural and veterinary chemicals. (Formerly) Standing Council on Environment and Water, February 2009, available at webarchive.nla.gov.au/gov/20141215101021/http://www.scew.gov.au/resource/chemical-risk-assessment-guidance-manuals.

Abbreviations

| AC | Active constituent |
| --- | --- |
| ACS | Active constituents |
| APVMA | Australian Pesticides and Veterinary Medicines Authority |
| BW | Body weight |
| DDD | Daily dietary dose |
| DDSD | Daily dry soil dose |
| DGD | Daily granule dose |
| DT50 | Period required for 50 per cent dissipation |
| DT90 | Period required for 90 per cent dissipation |
| EC50 | Effective concentration, median |
| ErC50 | Effective concentration, median, growth rate |
| EyC50 | Effective concentration, median, yield |
| EFSA | European Food Safety Authority |
| EPHC | Environment Protection and Heritage Council |
| EPPO | European and Mediterranean Plant Protection Organization |
| ER25 | Effective rate, 25th per centile |
| ER50 | Effective rate, median |
| EUBEES | European Union Biocides Environmental Exposure Scenario working group |
| FIR | Food Intake Rates |
| HR5 | Hazardous rate to 5% of the species |
| IPM | Integrated Pest Management |
| Kd | Adsorption constant |
| Koc | Organic carbon absorption coefficient |
| Kow | Octanol-water partition coefficient |
| LC50 | Lethal concentration, median |
| LD50 | Lethal dose, median |
| LOEC | Lowest observed effect concentration |
| LR50 | Lethal rate, median |
| MDR | Model deviation ratio |
| MRL | Maximum residue limit |
| NAR | Nominal (loading) application rate |
| NOAEL | No-observed-adverse-effect level |
| NOEC | No-observed-effect concentration |
| NOEL | No-observed-adverse level |
| OECD | Organisation for Economic Co-operation and Development |
| PAA | Pre-application Assistance |
| PBT | Persistent, bioaccumulative and toxic |
| PD | Composition of diet obtained from treated area |
| PEC | Predicted environmental concentration |
| pKa | Negative logarithm (to the base 10) of the dissociation constant |
| POP | Persistent organic pollutants |
| PT | Proportion of diet obtained from treated area |
| RAC | Regulatory acceptable concentration |
| RAD | Regulatory acceptable dose |
| RAL | Regulatory acceptable level |
| RAR | Regulatory acceptable rate |
| RQ | Risk quotient |
| USEPA | USA Environmental Protection Authority |
| VICH | International Cooperation on Harmonization of Technical Requirements of Veterinary Medicinal Products |

www.pops.int/Home/tabid/2121/Default.aspx

Predicted values calculated assuming additive toxicity of active constituents in a the specified ration (P1:P2) using most sensitive endpoints reported for that organism group where:

ECxCA=1i-1nPiECxi

Where:

ECxCA is the predicted additive toxic effect of the active constituent in combination

Pi = is the fraction of individual active constituent in the product

ECxi is the effect concentration of the individual active constituent

%relativeecotoxicitycontribution=piECxii=1npiECxi×100

MDR = model deviation ratio (unitless) = measured EC50 / predicted EC50, toxicity of combined residues is considered more than additive if MDR <0.2, additive if MDR 0.2-5, and less than additive if MDR >5